Technical Report on the Katowice City Sewage System

Glossary / Słownik Pojęć Kluczowych

| Polish Term | English Equivalent | Description |
| --- | --- | --- |
| Kanalizacja Sanitarna | Sanitary Sewerage | System for conveying domestic and industrial wastewater. |
| Kanalizacja Burzowa | Storm Drainage | System for conveying rainwater and meltwater. |
| Kanalizacja Ogólnospławna | Combined Sewerage | System conveying both sewage and stormwater. |
| Oczyszczalnia Ścieków (OŚ) | Wastewater Treatment Plant (WWTP) | Facility for cleaning wastewater. |
| Przepompownia Ścieków | Sewage Pumping Station | Facility essential for moving wastewater over varied terrain. |
| Redukcja Biogenów | Biogen Reduction | Removal of nitrogen and phosphorus compounds. |
| Inspekcja Telewizyjna (TV) | Television Inspection (TV Inspection) | Diagnostic method using cameras to check pipe condition. |
| Technologie Bezwykopowe | Trenchless Technologies | Methods used for pipe repair without extensive digging (e.g., relining). |
| Zbiorniki Retencyjne | Retention Reservoirs | Structures for temporarily storing excess stormwater. |
| Górniczy Charakter Terenu | Mining Terrain Character | Ground conditions affected by historical mining activity. |

Managing Entity: Katowickie Wodociągi S.A. (Katowice Waterworks S.A.) Basis of the Report: Analysis of operational, investment, and organizational data. Purpose of the Report: Detailed presentation of the technical status, organization, and development directions of the sewage infrastructure in Katowice.

I System Fundamentals and Operations Organization

1.Basic Infrastructure and Network Scope

The sewage system in Katowice constitutes an extensive, multifunctional infrastructure with a total operational length of over 860 kilometers. The network is characterized by a high diversity of materials (ranging from older technologies, such as concrete or stoneware, to modern materials) and is located in complex, mining terrain

Wastewater Treatment Plants

All collected sanitary wastewater (and wastewater from combined sewer systems) is directed to four main Wastewater Treatment Plants (WWTPs):

- Treatment Plant Gigablok

- Treatment Plant Dąbrówka Mała - Centrum

- Treatment Plant Panewniki

- Treatment Plant Podlesie

- Technology

- All these facilities are mechanical-biological-chemical treatment plants. The use of chemical technology is crucial because it enables the reduction of biogens (nitrogen and phosphorus), thereby meeting the rigorous European Union standards for the quality of water discharged into the environment.

- Capacity

- The total throughput of these treatment plants exceeds 103.5 thousand m³ of wastewater per day, which is sufficient to serve the needs of a large city and ensure ecological safety.

2. Functional devision of the network

It is crucial to distinguish and manage the two main system types separately, with particular attention to historical combined sections:

- Sanitary Sewerage: Responsible for conveying exclusively domestic, commercial, and industrial wastewater. In newly built and modernized areas, it is completely separated from stormwater drainage. Due to the varied terrain, it requires the use of numerous sewage pumping stations to transport sewage to main collectors and treatment plants.

- Storm Drainage: Used for conveying rainwater and meltwater. In the separate system, stormwater is directed straight to natural receivers, often after preliminary purification in separators. Challenges include sections of combined sewerage and the necessity for intensive development of retention infrastructure.

3. Organization of Operation and Maintenance

Network operation and maintenance are territorially divided, which optimizes reaction time to breakdowns resulting from the natural river catchment layout:

- Sewerage Network Operation Division - Centrum (Centre): Responsible for the northern, eastern, and central areas (including Śródmieście, Załęże, Bogucice).

- Sewerage Network Operation Division - Południe (South): Responsible for the southern and western areas (including Murcki, Piotrowice, Ligota - Panewniki, Kostuchna, Podlesie).

Both Divisions are responsible for sewer lines, breakdown removal, and the operation and maintenance of sewage pumping station

Diagnostics and Technical Condition of the Network

4. Methods for Assessing Technical Condition (Diagnostics)

Maintaining over 860 km of network in proper technical condition relies on advanced diagnostics, which form the foundation for renovation planning

| Diagnostic Method | Purpose and Application | Critical Importance |
| --- | --- | --- |
| Television Inspection (TV) | Systematic introduction of cameras into pipes, analysis of pipeline technical condition, identification of cracks, blockages, groundwater infiltration, and illegal connections | Identifying sections for repair/modernization and assessing investment priorities. |
| Slope measurements/profiling | Creating slope charts, assessing channel patency and capacity. | Diagnosing problems with gravity-based sewage transport. |
| Damage Classification | Developing detailed reports with descriptions and classification of damage (e.g., according to EN standards), used to select the repair method. | Basis for selecting trenchless renovation technology. |

5. Identified Technical Condition Challenges

- Material Aging: A significant portion of the network is constructed using older technologies (clayware, concrete), which show a higher damage rate per 1 km compared to modern plastics.

- Impact of Mining: The complex, mining-related nature of the terrain generates ground stresses and displacements, leading to cracks, shifts in pipe joints, and collector deformations. This necessitates continuous renovation and the use of flexible technologies.

- Groundwater Protection: Ensuring the network's tightness (sealing) is crucial, especially given the shallow depth of groundwater in parts of the city, to prevent infiltration (groundwater penetrating the sewage system) and exfiltration (sewage penetrating the soil).

6. Maintenance and Renovation Technologies

Network maintenance is carried out through current failure removal and planned renovations. The priority is the use of trenchless technologies (renovation), which minimize inconvenience for residents and costs associated with disturbing road surfaces. Modernization serves not only for repair but also for the rationalization of sewage discharge and the elimination of illegal connections that burden the treatment plants with rainwater.

Sewage Treatment System and Stormwater Management

7. Sewage Treatment Effectiveness

Four mechanical-biological-chemical treatment plants guarantee an ecologically safe process:

- Technology: The use of chemical processes is essential for the enhanced treatment of biogens (nitrogen and phosphorus reduction), allowing for compliance with the requirements of the Water Framework Directive and the minimization of impact on natural receiving waters (mainly the Kłodnica and Rawa rivers).

- Modernizations: Facilities, such as the Panewniki Treatment Plant (modernized in 2012), undergo cyclical modernizations to maintain efficiency and adapt to evolving regulations. The achieved capacity (> 103.5 thousand m³/day) ensures a reserve for the city's future development.

8. Role and Condition of Sewage Pumping Stations

Sewage pumping stations are a sensitive but critical element of the system, necessary for overcoming terrain elevation differences:

- Critical Role: They ensure the continuous pumping of sewage to high-lying treatment plants, which is essential given the highly varied topography of Katowice.

- Management: Their operation, maintenance, and planned modernization (e.g., the pumping station at Marcina Street) are a constant part of investment plans. Modernization work focuses on replacing pumps, control systems, and power supply to adapt them to increasing loads and ensure resistance to failures. The exact, current number of these facilities is not publicly disclosed, but their network is extensive.

9. Challenges Related to Stormwater and Retention

Cloudbursts have necessitated significant investments in the stormwater system, which is managed in cooperation with the City Council:

- Anti-Flooding Investments: Projects are being implemented to increase the city's retention capacity through the construction of new stormwater pumping stations and the installation of open and underground retention reservoirs (e.g., in the area of Korczaka, Andersa, Filarowa, Grodowa streets).

- Retention System: These reservoirs temporarily store excess water, which is crucial for relieving the burden on the storm sewers and minimizing the risk of urban flooding, and then allow for its gradual discharge into watercourses.